KM5201

MPEG-4 Video Decoder

ADVANCE INFORMATION

KM5201 is a highly integrated silicon core for MPEG-4 Video Decoder. It supports Simple Profile, Levels 1 to 3 for QCIF (176x144)  and CIF (352x288)  resolution video  decoding with low bit rates up to 384  Kb/s. Its open architecture allows direct interface with other IP cores like embedded processors, USB, and VGA etc., to build complete System-on-a-chip (SoC) solutions.

K2 Media Networks, Inc.

Features

- Compliant with ISO/IEC 14496-2 standard video decompression

- Supports Simple Profile Levels 1-3

- All the features required are built into the core

- Supports I-VOPs and P-VOPs

- IEEE-1180 compliant IDCT

- Type 1 dequantizer

- Inverse DC/AC prediction

- Fully synchronous design

- Low power design for portable devices

- Generic host PIO interface

- Generic  DMA interface to SDR/DDR

- Low latency embedded processor cache bus interface

- Internal low power 16-bit SDR/DDR  bus

- YCbCr  4:2:0 raster stream output

Optional Features

- YCbCr to RGB conversion

- Video scaler to convert  to VGA or D1 resolution from CIF or QCIF

- Type 2 quantizer  support (H.263+)

- H.263+ compatible stream decode

- Overlapped block motion compensation filter

- Deblocking/deringing post processing filter

- Resynchronization features

MB

Buffer

Motion

Compensation

DMA

I/F

Host

I/F

Bit Stream

Parser

Inverse

DC/AC Prediction

Inverse Quant

Inv. Scan

Inverse

DCT

PIO Bus

DMA Bus

Memory Arbiter

SDR/DDR Controller

Proc.

Cache

I/F

Functional Description

A compatible MPEG4 Video elementary stream can be transferred from the external DMA interface to SDR/DDR memory. All the core registers are programmable from the external host PIO interface to start the decoder operation. The bit stream parser block takes data from SDR/DDR buffers and parses the bit stream into decompression parameters and compressed macro block data. The compressed macro block data is inverse scanned, inverse dc/ac prediction performed, dequantized, and sent through IDCT block. For P-VOPs motion compensation is performed on a parallel data path and the prediction is added to the incoming stream.  The decoded data is then stored into SDR/DDR in YCbCr 4:2:0 raster scan order for further processing or display. The following simple profile table illustrates the compatible stream parameters.

Parameters at different levels of Simple Profile as specified in ISO/IEC 14496-2 are:

| Level | Typical Visual Session Size | Macro blocks per VOP | Maximum VBV Buffer Size | Maximum number of objects | Maximum Video Packet length (bits) | Maximum Bit rate (kbps) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | QCIF | 99 | 10 | 4 | 2048 | 64 |
| 2 | CIF | 396 | 40 | 4 | 4096 | 128 |
| 3 | CIF | 396 | 40 | 4 | 8192 | 384 |

Table 1: Parameters of Simple Profile

## Applications

• Digital video playback devices

• Video streaming decoder devices

• Portable (PDA, Cell Phone) video devices

• Wireless video devices

## Deliverables

• RTL source code (Verilog) or optimized net list

• Bit-accurate C model

• Test bench

• Documentation