KM5202

MPEG-4 Video Encoder

ADVANCE INFORMATION

K2 Media Networks, Inc.

KM5202 is a highly integrated silicon core for MPEG-4 Video Encoder. It supports Simple Profile Levels 1 to 3 for QCIF (176x144) and CIF (352x288) resolution video encoding with low bit rates up to 384 Kb/s.  Its open architecture allows direct interface with other IP cores like embedded processors, USB, and VGA etc., to build complete System-on-a-chip (SoC) solution.

Features

- Compliant with ISO/IEC 14496-2 standard video compression

- Supports Simple Profile Levels 1-3

- All the features required are built into the core

- Supports I-VOPs and P-VOPs up to 4 video objects

- Supports motion vectors +/- 16 search range

- Type 1 quantizer

- DC/AC intra prediction

- Fully synchronous design

- Low power design for portable devices

- CCIR 601 Y Cr Cob interface

- Generic host PIO interface

- Generic  DMA interface to SDRAM

- Low latency embedded processor cache bus interface

- Internal 16-bit SDRAM bus

- Interface to 16Mbit or 64Mbit SDRAM

- YCbCr  4:2:0 raster stream output

Motion Comp.

DMA

I/F

Host

I/F

Entropy Coder

Inverse Quant

Inverse

DCT

PIO Bus

DMA Bus

Memory Arbiter

SDRAM Controller

Video

I/F

Proc.

Cache

I/F

DCT

Quant

Scan

Motion Est.

DC/AC Prediction

Optional Features

- Video scaler to convert  CCIR 601 resolution to CIF or QCIF

- Type 2 quantizer Support (H.263+)

- H.263+ compatible stream encode

- Resynchronization Features

Functional Description

The CIF or QCIF YCbCr 4:2:0 video resolution data can be transferred from the external DMA interface to SDRAM. All the core registers are programmable from the external host PIO interface to start the encoder operation. Incoming video data is converted into macro blocks and coded as I-VOP or P-VOP based on register parameters and bit rate feed back. The intra macro blocks are passed through DCT transform, quantizer, dc/ac prediction, scan, and entropy coder modules.

The inter blocks are passed through motion estimation, DCT transform, quantizer, scan, and entropy coder modules. The resultant encoded bit stream is stored into the SDRAM for further processing or storage. The following simple profile table illustrates the stream parameters.

Parameters at different levels of Simple Profile as specified in ISO/IEC 14496-2 are:

| Level | Typical Visual Session Size | Macroblocks per VOP | Maximum VBV Buffer Size | Maximum number of objects | Maximum Video Packet length (bits) | Maximum Bitrate (kbps) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | QCIF | 99 | 10 | 4 | 2048 | 64 |
| 2 | CIF | 396 | 40 | 4 | 4096 | 128 |
| 3 | CIF | 396 | 40 | 4 | 8192 | 384 |

Table 1: Parameters of Simple Profile

## Applications

• Digital video capture and storage devices

• Video conferencing

• Surveillance video

• Solid-state video cameras/camcorders

• Video recorders

• Video streaming devices

• Portable (PDA, Cell Phone) video devices

• Wireless Video devices

## Deliverables

• RTL source code (Verilog) or optimized net list

• Bit-accurate C model

• Test Bench

• Documentation