KM5204

JPEG-Like Video Encoder

ADVANCE INFORMATION

K2 Media Networks, Inc.

KM5204 is a highly integrated silicon core for JPEG-like video encoding for still and motion video. It supports still frame mega pixel compression and VGA and CIF resolution video encoding at 30 frames/sec. It also supports video preprocessing functions such as windowing, video format conversion and continuous down scaling to any resolution besides video encoding. A rate control module is also available that continuously monitors video buffers and adjusts compression for motion video.  Its open architecture allows direct interface with other IP cores like embedded processors, USB, and VGA etc., to build complete System-on-a-chip (SoC) solution.

Features

- Mostly based on JPEG compression standard.

- Additional features included to reduce the gate count and memory requirements.

- All the features required are built into the core.

- Supports Still or Motion Video.

- Separate Quantizers for Y and Cr Cb.

- DC intra prediction

- Fully synchronous design

- Low power design for portable devices

- CCIR 601 Y Cr Cb  like interface.

- Generic host PIO interface

- Optional Generic  DMA interface to SDRAM

- Optional Low latency embedded processor cache bus interface

- Internal 16-bit SDRAM bus

- Interface to 16Mbit or 64Mbit SDRAM

- YCbCr  4:2:0 raster stream compression

DMA

I/F

Host

I/F

PIO Bus

DMA Bus

Memory Arbiter

SDRAM Controller

Video

I/F

Proc.

Cache

I/F

DCT

Quant

Scan

Rate Control

Entropy Coder

DPCM & RLC

window & scale

Buffer

Functional Description

Video I/F accepts data in any digital video format (YUV 4:4:4 etc.,) and performs preprocessing functions such as windowing, video format conversion, and downscaling to required resolution for compression. Video data in YUV 4:2:0 is forwarded to strip buffers (only 8 lines of Y and 8 lines of memory for CrCb is required)  for intermediate storage and block conversion. The 8x8 blocks of Y, Cr Cb are compressed with DCT, quantized with their  respective quantizers, run length coded. The RLC data is entropy coded and forwarded to compressed video buffers.  The rate control device monitors the buffer levels and controls various compression parameters for bit-rate constrained applications.

## Applications

• Digital video capture and storage devices

• Video conferencing

• Surveillance video

• Solid-state video cameras/camcorders

• Video recorders

• Video streaming devices

• Portable (PDA, Cell Phone) video devices

• Wireless Video devices

## Deliverables

• RTL source code (Verilog) or optimized net list

• Bit-accurate C model

• Test Bench

• Documentation

## Core

• Compression core is only 20-25K gates

• Required only 16 lines of memory for strip buffer

• Other memories are 64x16 transpose buffer and VLC tables

• Fully pipelined design.