KM5208

H.264/AVC/MP4 Video Decoder Decoder

ADVANCE INFORMATION

KM5208 is a highly integrated silicon core for H.264/AVC/MPEG-4 Part-10 video decoder. It supports Baseline  Profile, Levels 1 to 3 for QCIF (176x144)  and CIF (352x288)  and D1 video (720x576) resolution video  decoding with low bit rates from 64  Kb/s to up to  10 Mbits/s. Its open architecture allows direct interface with other IP cores like embedded processors, USB, and VGA etc., to build complete System-on-a-chip (SoC) solutions.

K2 Media Networks, Inc.

Features

- Compliant with ISO/IEC 14496-10 standard video decompression

- Supports Baseline Profile Levels 1-3

- All the features required are built into the core

- Supports I,  P Frames

- Fully synchronous design

- Low power design for portable devices

- Generic host PIO interface

- Generic  DMA interface to SDRAM

- Low latency embedded processor cache bus interface

- Internal 16-bit SDRAM bus

- Interface to 16Mbit or 64Mbit SDR/DDR

- YCbCr  4:2:0 raster stream output

Optional Features

- YCbCr to RGB conversion

- Video scaler to convert  to VGA or D1 resolution from CIF or QCIF

- Error-Resilence  features

MB

Buffer

Motion

Compensation

DMA

I/F

Host

I/F

Bit Stream

Parser

Inverse Quant

Inv. Scan

Inverse

Transform

PIO Bus

DMA Bus

PIO/ Memory Arbiter

SDR/DDR Controller

Proc.

Cache

I/F

Deblocking Filter

Functional Description

A compatible H.264/AVC/MPEG4 Part-10 baseline video stream can be transferred from the external DMA interface to SDR/DDR. All the core registers are programmable from the external host PIO interface to start the decoder operation. The bit stream parser block takes data from SDR/DDR buffers and parses the bit stream into decompression parameters and compressed macro block data. The compressed macro block data is inverse scanned, dequantized, and sent through Inverse Transform block. For P-VOPs motion compensation is performed on a parallel data path and the prediction is added to the incoming stream.  The decoded data is passed through deblocking filter before getting stored into SDR/DDR in YCbCr 4:2:0 raster scan order for further processing or display. The following simple profile table illustrates the compatible stream parameters.

Parameters at different levels of Baseline Profile  supported as specified in ISO/IEC 14496-10 are:

Table 1: Parameters of Baseline Profile

| Level number | Max macroblocks per second | Max frame size (macroblocks) | Max video bit rate (VCL) for Baseline, Extended and Main Profiles | Max video bit rate (VCL) for High Profile | Max video bit rate (VCL) for High 10 Profile | Max video bit rate (VCL) for High 4:2:2 and High 4:4:4 Predictive Profiles | Examples for high resolution @ frame rate (max stored frames) in Level |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1485 | 99 | 64 kbit/s | 80 kbit/s | 192 kbit/s | 256 kbit/s | 128x96@30.9 (8) 176x144@15.0 (4) |
| 1b | 1485 | 99 | 128 kbit/s | 160 kbit/s | 384 kbit/s | 512 kbit/s | 128x96@30.9 (8) 176x144@15.0 (4) |
| 1.1 | 3000 | 396 | 192 kbit/s | 240 kbit/s | 576 kbit/s | 768 kbit/s | 176x144@30.3 (9) 320x240@10.0 (3) 352x288@7.5 (2) |
| 1.2 | 6000 | 396 | 384 kbit/s | 480 kbit/s | 1152 kbit/s | 1536 kbit/s | 320x240@20.0 (7) 352x288@15.2 (6) |
| 1.3 | 11880 | 396 | 768 kbit/s | 960 kbit/s | 2304 kbit/s | 3072 kbit/s | 320x240@36.0 (7) 352x288@30.0 (6) |
| 2 | 11880 | 396 | 2 Mbit/s | 2.5 Mbit/s | 6 Mbit/s | 8 Mbit/s | 320x240@36.0 (7) 352x288@30.0 (6) |
| 2.1 | 19800 | 792 | 4 Mbit/s | 5 Mbit/s | 12 Mbit/s | 16 Mbit/s | 352x480@30.0 (7) 352x576@25.0 (6) |
| 2.2 | 20250 | 1620 | 4 Mbit/s | 5 Mbit/s | 12 Mbit/s | 16 Mbit/s | 352x480@30.7(10) 352x576@25.6 (7) 720x480@15.0 (6) 720x576@12.5 (5) |
| 3 | 40500 | 1620 | 10 Mbit/s | 12.5 Mbit/s | 30 Mbit/s | 40 Mbit/s | 352x480@61.4 (12) 352x576@51.1 (10) 720x480@30.0 (6) 720x576@25.0 (5) |

## Applications

• Digital video playback devices

• Video streaming decoder devices

• Portable (PDA, Cell Phone) video devices

• Wireless video devices

## Deliverables

• RTL source code (Verilog) or optimized net list

• Bit-accurate C model

• Test bench

• Documentation